MATERIAL SAFETY DATA SHEET

OF C5 HYDROCARBON RESIN

COMPANY NAME：THANKYOU CHEM (HENAN) CO.,LTD.

ADD.: NO.231,NORTH OF JINGKAI STREET, PUYANG CITY, HENAN PROVINCE, CHINA

TEL：86-393-8110188     FAX：86-393-8110366    CELL：86-13903930633

WEB : www.thankyouchem.com         E-MAIL：export@thankyouchem.com

P.C.: 457000                         CONTACT： MR. ZISEN GAO

SECTION 1:CHEMICAL NAME: C5 HYDROCARBON RESIN, CAS NUMBER: 64742-16-1.

SECTION 2: COMPOSITION OR INFOMARTION ON INGREDIENTS:

COMPONENT: HYDROCARBON RESIN; IT IS MIXTURE, THE MAIN RAW MATERIAL IS 1’3PENTADIENE, WHICH IS BY-PRODUCTS OF HYDROCARBON’S FURTHER CRACKING..

SECTION 3: HAZARD IDENTIFICATION: 1.DETERMINATION OF TOXIC & INFECTIONS SUBSTANCES, THE SUBSTANCE DOES NOT BELONG TO TOXIC SUBSTANCES; 2.DETERMINATION OF RADIOACTIVE MATERIALS, THE SUBSTANCE DOES NOT BELONG TO RADIOACTIVE MATERIALS; 3. DETERMINATION OF CORROSIVES, THE SUBSTANCE HAS NO OBVIOUS HARM TO SKIN AND METAL, SO IT DOES NOT BELONG TO CORROSIVE; 4. DETERMINATION OF OTHER DANGEROUS PROPERTIES, THE SUBSTANCE HAS NO OTHER DANGEROUS PROPERTIES; 5. NFPA RATINGS<SCALE0-4>:
HEALTH =0 FIRE =1 REACTIVITY=0; 6.CARCINOGENICITY :NOT LISTED AS CARCINOGEN BY NTP ,OSHA AND IARC.
SECTION 4:FIRST-AID MEASURES: IN CASE OF SKIN CONTACT, FLUSH WITH SOAP AND WATER; IN CASE OF EYES CONTACT, FLUSH WITH FLOWING WATER; IF SWALLOWED, GET MEDICAL AID.

SECTION 5:FIRE-FIGHTING MEASURES: FLAMMABILITY CLASS (OSHA):IIIB; THERMAL DECOMPOSITION PRODUCTS MAY INCLUDE TOXIC OXIDES OF CARBON; USE WATER FOG, CARBON DIOXIDE,DRY CHEMICAL POWDER, FOAM, EXTINGUISHER OR SANDY SOIL FOR EXTINGUISHING FIRE.

SECTION 6:ACCIDENTAL RELEASE MEASURES: IN CASE OF SPILL, SWEEP, SCOOP OR VACUUM AND REMOVE, STORE INTO A DRY, CLEAN LIDDED CONTAINER FOR DISPOSAL.

SECTION 7: HANDING & STORAGE: KEEP IN A TIGHTLY CLOSED CONTAINER, PROTECT FROM PHYSICAL DAMAGE. STORE IN COOL, VENTILATED PLACE. KEEP AWAY FROM HEAT, SPARKS OR OPEN FLAME. KEEP AWAY FROM STRONG OXIDIZING AGENTS, ACIDS AND BASES.

SECTION 8:ACCESS CONTROL/PERSONAL PROTECTION: WEAR APPROPRIATE PROTECTING CLOTHING, GLASSES AND SAFETY GLOVES.

SECTION 9:PHYSICAL & CHEMICAL CHARACTERISTICS: APPEARANCE AND ODOR:YELLOW GRANULE SOLID, ODORLESS, HYDROCARBON ODOR; INSOLUBLE IN WATER, HARDLY SOLUBLE IN ACETONE AND ETHANOL; WHEN THE MELTING POINT MORE THAN 60ºC, IT WILL BEGIN TO SOFTEN; THE COMPARATIVE DENSITY IS BETWEEN 0.83 AND 1.12; FLASHING POINT IS ABOUT 220ºC.

SECTION 10: STABILITY AND REACTIVITY: STABLE UNDER NORMAL STORAGE CONDITION, THE SUBSTANCE DOES NOT REACT VIGOROUSLY WITH REDUCING AGENT, SO IT DOES NOT BELONG TO OXIDIZING SUBSTANCE; PROHIBITION CONTACT WITH STRONG OXIDIZING AGENTS; HAZARDOUS DECOMPOSITION OR BY PRODUCTS: CARBON MONOOXIDE, CARBON DIOXIDE; AND WILL NOT OCCUR HAZARDOUS POLYMERIZATION.

SECTION 11:DETERMINATION OF FLAMMABILITY: THE BURNING RATE TEST IS CONDUCTED IN ACCORDANCE WITH THE IATA DGR, THE RATE OF BURNING IS LESS THAN 2.2MM/S, SO THE SUBSTANCE DOES NOT BELONG TO FLAMMABLE.

SECTION 12:DETERMINATION OF EXPLOSIVES: THE SUBSTANCE PRESENTS NO EXPLOSIVES HAZARD.

SECTION 13:WASTE DISPOSAL: IT BELONGS TO INDUSTRIAL SOLID WASTE; PLACE INTO A DRY, CLEAN LIDDED CONTAINER FOR DISPOSAL.

SECTION 14:THE FILE IS ACCORDING TO IMO REGULATION. THEY ARE NON-HAZARDOUS PRODUCTS; HOWEVER, THE DIRECT SUNSHINE AND RAIN SHALL BE AVOIDED DURING TRANSPORTATION. THEY CAN’T BE TRANSPORTED TOGETHER WITH SELF-FLAMMABLE MATERIALS, STRONG OXIDIZERS AND STRONG ACID. THE PACKAGES ARE PLASTIC BAG OR PP BAG OR PAPER BAG.

SECTION 15:REGULATORY INFORMATION: PRODUCT QUALITY SPECIFICATION IN ACCORDANCE WITH P.R.C. CHEMICAL INDUSTRY STANDARD HG2231-91.

SECTION 16:OTHER INFORMATION：THE FILE IS JUST FOR YOUR REFERENCE. PLEASE HAVE YOUR BEST CHOICE IN ACCORDANCE WITH PRODUCTS’ USE METHODS AND SUITABLILITY. BOTH THE COMPANY AND THE PERSON WHO WRITES IT  HAVE THE FINAL EXPLANATION RIGHTS.